Project Status Meeting Protocol

Date: 02/04/2022

Topic: Drive system for the closed bore Hall probe measurements.

Attendees: Heinz-Dieter Nuhn, Zack Wolf, Yurii Levashov

Presented by: Yurii Levashov

Summary:

A custom system based on Parker gantry system #4 (Parker catalog) was presented. The system is a stand-alone belt-driven positioner and provides 5m long smooth motion of a carriage. The system uses Ethernet TCP/IP smart motor with built-in controller. The vertical stage was removed from the design.

Recommendations:

- Add extra rail for rigidity to minimize roll.

- Ask vendor for “smoothness” data.

- Consider adding extra supports to minimize rail sag.

- Add extra “idler” carriage for carbon tube support.

- Add encoder for triggering option.

- Consider using a stepper motor and an existing controller to minimize future programming efforts.

Actions:

The system will have two rails, one actuator and one idler, with an extra carriage, 5um resolution magnetic encoder, and 4 vertical supports. No data received from vendor about motion smoothness.

A quote received. The system is ready to be ordered.

Protocol prepared by: Yurii Levashov

Date: 02/23/2022