NON-COMPLIANCE REPORT # 31732-35

| PART NAME: | SUPPORT COLLAR | DATE: | 4/29/15 |  |  |
| --- | --- | --- | --- | --- | --- |
| PF/SA NO.: | PF-391-738-71 | INSPECTOR: | K. CABAN |  |  |
| WORK ORDER NO.: | 31732-35 | ENGINEER: | JC | EXT.: | 8901 |
| VENDOR | BT LASER | ROUTE TO: |  |  |  |

| Reason for Non-Compliance: | Disposition (UAI, Rework, Scrap, Other): |
| --- | --- |

- All  holes are oversized ranging from 0.2506 – 0.2509 (+0.0001 – 0.0004” out of tolerance).

- All  ± 0.02 radii are undersized ranging from 0.1496 – 0.1509 (-0.0191 - -0.0204” out of tolerance).

- of  is out of true position to Datum’s A & B measuring 0.0091 – 0.0110 (+0.0041 - +0.0060”
out of tolerance).

- of  is out of true position to Datum’s A-B-C measuring 0.0057 – 0.0075.

- of  is out of true position to Datum’s A-B-C measuring 0.0033 – 0.0044.