NON-COMPLIANCE REPORT # M-24760

| PART NAME: | MALE SKARPAAS TRANSITION FLANGE | DATE: | 1/26/15 |  |  |
| --- | --- | --- | --- | --- | --- |
| PF/SA NO.: | PF-701-267-08 R0 | INSPECTOR: | K. CABAN |  |  |
| WORK ORDER NO.: | M-24760 | ENGINEER: | A. KRASNYKH | EXT.: | 2444 |
| VENDOR | MFD/KLYSTRON MACHINE SHOP | ROUTE TO: | KLY |  |  |

| Reason for Non-Compliance: | Disposition (UAI, Rework, Scrap, Other): |
| --- | --- |

10 out of 14 parts were rejected out of tolerance.

- of length 2.845 is out of position on 4 parts (SN 111, 114, 115 & 118) ranging from
0.0021” to 0.0028”.

- of length 1.345 is out of position on 10 parts (SN 107, 108, 111, 114, 115, 116, 117, 118, & 120
ranging from 0.0030” to 0.0101”.

- of surface .187 is out of parallelism on 4 parts (SN 111, 116, 118 & 120) ranging from 
0.0026” – 0.0030”.

- of surface .627 is out of parallelism on 6 parts (SN 107, 113, 114, 116, 118 & 120) ranging
from 0.0021” to 0.0031”