How to recover from E-Stop (HXU)

- At “ID System Manager Control”, click “Other controls” tab and choose “FPGA Status Information”

- Click “Calculate Offsets”

- Latch FPGA to “Low” and to “High”

- Cycle “Motor Power” OFF/ON

- Go back to “ID System Manager Control” window and click “Calibrate Motors” (needs to be done if “Move Motor” numbers do not agree with “Enc=>Motor” numbers)