## SLAC Traveler for the LCLS ‘1.259Q3.5’ BSY Quadrupole Magnet QSM1

(May 2, 2014)

This traveler covers magnetic measurements of the LCLS “1.259Q3.5” BSY quadrupole magnet QSM1.  This quadrupole magnet is 0.108 m long, with a 0.032 m diameter.

Account # 7720080

Receiving:

The following information is to be noted upon receipt of the magnets by the SLAC MFD group:

| Received by (initials): | SDA |
| --- | --- |
| Date received (dd-mmm-yyyy): | 5/1/2014 |
| SLAC barcode number: 421 | 000421 |
| Serial number on the magnet: 22 | 22 |

| Place duplicate the barcode sticker here |  |
| --- | --- |

Preparation:

Verify that a beam direction arrow is in place applied to the top and/or connector side of the magnet.

| Beam-direction arrow in place (initials): | SDA |
| --- | --- |

Fiducialization:

Fiducialization was performed in 2006 and is deemed adequate.

URL of on-line Alignment fiducialization data (please modify or correct if necessary):

\\web002\www-group\met\Quality\FIDUCIAL REPORTS\LCLS QUADS\LCLS QUAD 000421.pdf

Magnetic Measurements:

Enter URL of on-line magnetic measurements data (please modify or correct if necessary):

http://www-group.slac.stanford.edu/met/MagMeas/MAGDATA/LCLS/skew_quad/QSM1/

- QSM1 is a “QF” (positive polarity”) skew quad.

| Magnet marked as (please enter “QF”): | SDA |
| --- | --- |

- Determine the connection polarity (with supply outputting positive current) which produces the correct field polarity for the positive or QF skew magnet as shown in Figure 1.

Figure 1.  QSM1 is a positive skew quad.  A “positive” skew quadrupole magnet is simply a 45° clockwise-rotated (around z axis) normal “positive” quadrupole.

- Mark the polarity near the magnet leads with clear “+” and “” labels as shown above.

| Polarity has been labeled (initials): | SDA |
| --- | --- |

- Connect the magnet terminals in the correct polarity as established above, to a bipolar power supply with maximum current I  +/-12 A.

- Run the magnet up to 8 A for ~30 minute to warm it up (record temperature).

| Ambient temperature (°C): | 22.8 °C |
| --- | --- |
| Final magnet temperature (°C): | 24.8 °C |

- Standardize the magnet, starting from -12 A to +12 A through three full cycles, finally ending at -12 A, with a flat-top pause time (at both -12 A and +12 A) of 10 seconds.  Use a ramp rate of 1 A/sec, and ramp style three-linear, and record the ramp rate and ramp style used.

| Standardization complete (initials): | SDA |
| --- | --- |
| Ramp rate used (A/sec): | 1 A/sec |

- Measure the length-integrated field gradient, Gdl, from -12 A to +12 A to -12 A in 1-A steps, and then back down from +12 A to -12A in 1-A steps (49 measurements total).

| Filename & run number of Gdl up & down data: | Strdat.ru1 |
| --- | --- |

- Confirm the pole-tip field using a Hall probe at an excitation current of 4 A.

| Hall probe pole-tip field at 4 A (mean of 4 poles): | 0.097 Tesla at 4.00255 Amps |
| --- | --- |

- Measure the field harmonics at a A current setting using a 0.8-inch diameter probe.

| Rotating coil designation (coil name): | 1_124DQB22_4_Layer |
| --- | --- |
| Rotating coil radius (m): | 0.01402 m |
| Harmonics data file name: | Hardat.ru1 |

- Measure the inductance and resistance of the magnet:

| Inductance of coil (mH): | 9.72 mH @ 1000 Hz |
| --- | --- |
| Resistance of coil (Ohms): | 0.4005 Ohm at 12 Amps |

- Standardize the magnet, starting from -12 A to +12 A through three full cycles, finally ending at -12 A, with a flat-top pause time (at both -12 A and +12 A) of 10 seconds.  Use a ramp rate of 1 A/sec, and ramp style three-linear, and record the ramp rate and ramp style used.

| Standardization complete (initials): | SDA |
| --- | --- |
| Ramp rate used (A/sec): | 1 A/sec |

- Establish optimum de-gauss cycle to achieve a remnant field of less than 0.02 kg.  Record the current set points, number of cycles, ramp rate and ramp style used.

| De-gauss complete (initials): | SDA |
| --- | --- |
| Mean integral remnant field after de-gauss: | 0.00055 kg |
| Ramp rate used (A/sec): | 10 A/sec |
| Ramp Wait (sec) | 2 sec |
| Ramp Style | Three Linear |
| Data File(s) | strdat.ru2 and strdat.ru3 |

- Upon completion of tests, attach the traveler to the magnet and send a copy to Alev Ibrahimov at mailstop 51 and to Rick Iverson at mailstop 51.

This section is to be completed by R. Iverson.

| Magnet accepted (signed): |  |
| --- | --- |
| Assigned beamline location (MAD-deck name): | QSM1 |