Step-by-step procedure to install a Carrier on the Strongback

- Clean-up surfaces

- Start with the carrier #2 (center).

- Using rail matching table, find appropriate V-rail and M-rail for bottom of the carrier, record the quadrant # and carrier # into the table.

- Attach end plates to the both sides of the rails.

- Clean-up the mating surfaces on the V-rail.

- Wet two 0.001” shims with alcohol and place them on the lower part of the strongback.

- Place the V-rail on the strongback and secure it with 2 M8x50 screws at the ends. Make the screws hand-tight.

- Place the roller cassette on the rail.

- Clean the M-rail up and place it on top of the V-rail. Move the M-rail back and forth to let rollers find the right position.

- Set-up the carrier on a cart, attach the lifting fixture to it.

- Clean the top mating surface with alcohol.

- Using rail matching table, find appropriate M-rail and V-rail for top of the carrier, record the quadrant # and carrier # into the table.

- Clean-up the mating surfaces on the M-rail, place it on the carrier and secure with 10 M8x45 screws. Make the screws hand-tight.

- Place the roller cassette on the rail.

- Clean up the V-rail and place it on top of the M-rail. Attach end plates to the rails on one side only. To the other side attach the rail motion restraining plate.

- Install rail pre-loading screw. The screws should be ~ 0.6” above the strongback surface.

- Wet two 0.001” shims with alcohol and place them on the top part of the strongback.

- Have load positioner on the hook.

- Lift carrier with the crane.

- Position the carrier by crane next to the strongback, ~1/4” above the target location. Use the load positioner to lower the carrier and push it into the place.

- Insert 2 M8X50 screws into the holes next to the shims and hand-tight secure the top rail on the strongback. Move carrier back and forth slightly to make holes into the right place.

- Install 10 M8X45 screws on the front lower part of the carrier. Make them hand-tight.

- Disconnect the lifting fixture.

- Replace the rail motion restraining plate with the end plates.

- Move the carrier back and forth to set the rollers.

- Torque the rail pre-loading screws to 5 N-m.

- Torque all 20 screws on the frond and 4 screws on the back to 5 N-m.

- Slightly (1/16 turn or less) lose the pre-load screws.

- Slightly (1/16 turn or less) lose the screws on the front and back.

- Re-torque pre-load screws to 5N-m in zig-sag pattern.

- Re-torque front and back screws to 5N-m in zig-sag pattern.

- Check the carrier motion by moving it back and forth. Leave it in the middle. The carrier #2 has being installed.

- Do items 1 to 5 for Carrier #1.

- Wet two 0.025” shims with alcohol and place them on the lower part of the strongback.