Step-by-step procedure to remove a Carrier from Strongback

- Take off earthquake brackets.

- Take off cam mover bracket on upstream side.

- Remove quadrant motion restrain bracket.

- Attach lifting fixture to the carrier.

- Have load positioner on the hook.

- Hook-up carrier to the crane and apply light tension to the strap.

- Attach end plate to the top rails.

- Remove carrier connection bolts.

- Remove 10 carrier - low rail bolts on the front side.

- Lose 4 carrier – rail bolts on the back by ½ turn.

- Lose pre-load screws by 1 turn.

- Remove 2 top carrier – rail bolts on the back.

- Use load positioner to lift the carrier a little bit up, so, it should easily come out of the strongback.

- Move the carrier by the crane and put it on a cart.

- Remove rail end plate.

- Take off the V-rail from the carrier, wrap into paper, mark it with rail # and Carrier # and top/bottom.

- Put the roller cassette aside.

- Remove M-rail from the strongback, wrap into paper; mark it with rail # and Carrier #.

- Put the roller cassette aside.

- Take off screws and remove V-rail from the strongback, wrap into paper, mark it with rail # and Carrier # and top/bottom.

- Collect and inspect shims.